Coursework 3 Report

Usman Ali

  - Calculate Focal Length

The focal length was calculated using the width of the camera sensor size and dividing this by width of the resolution. The resulting number was then multiplied by the focal length in pixels.

Fmm = Fpx x (Sw / Iw)

Fmm = 5806.559 x (22.2 / 3088)

Fmm = 41.74 mm (2dp)

1.2 	Disparity Map

Figure 1 Disparity Image with blocksize of 5 and numOfDisparities at 64

Figure 2 Disparity Image with blocksize of 15 and numOfDisparities at 64

Figure 3 Disparity Image with blocksize of 5 and numOfDisparities at 16

Figure 4 Disparity Image with blocksize of 5 and numOfDisparities at 128

The blockSize defines the size of the comparison window used to find the corresponding points between the left and right images. The robustness of disparity calculations can be enhanced by increasing the blockSize because more pixels are needed to establish correspondence. The result of this change can be seen when comparing Figure 2 with Figure 1. Figure 2 has a larger blockSize, and as a result the details in the image, represented by the thicker lines, are a lot smoother and there is less noise present. Although a smaller blockSize can retain more detail and identify edges and small objects more accurately, it may struggle in low-textured areas and be more vulnerable to noise.

The number of disparities represents the number of horizontal shifts that the algorithm will test to find correspondences between the left and right images. The depth range that the system is able to detect is directly affected by this parameter. A comparison between Figure 3 (disparity parameter set at 16) and Figure 4 (disparity parameter set at 128) allows us to see the effect of increasing the number of disparities. This increase results in a disparity map that is more detailed and captures a wider range of depth. In contrast, the disparity map seems less detailed and only the most significant shifts are depicted when the number of disparities is set to a smaller value, such as 16. These larger or closer objects typically have more pronounced depth variations from the background.

Figure 5

Figure 6

Figure 7

Figure 8

depth = 1/ (disparity + k)

In the depth calculation formula, the variable “k" serves as a control to avoid division by zero and to adjust how disparity changes affect the depth values that are obtained. A smaller k value increases the depth estimation's responsiveness to disparity alterations, a greater k value ensures a smoother depth map and mitigates noise.

Appendix

import numpy as np

import cv2

import sys

from mpl_toolkits import mplot3d

from matplotlib import pyplot as plt

# ================================================

def getDisparityMap(imL, imR, numDisparities, blockSize):

stereo = cv2.StereoBM_create(numDisparities=numDisparities, blockSize=blockSize)

disparity = stereo.compute(imL, imR)

disparity = disparity - disparity.min() + 1 # Add 1 so we don't get a zero depth, later

disparity = disparity.astype(np.float32) / 16.0 # Map is fixed point int with 4 fractional bits

return disparity # floating point image

# ================================================

# ================================================

def plot(disparity, f, cx, cy, baseline, doffs):

h, w = disparity.shape

x, y = np.meshgrid(np.arange(w), np.arange(h))

# Calculate Z, X, and Y for each pixel

Z = (baseline * f) / (disparity.astype(np.float32) + doffs)

X = (x - cx) * Z / f

Y = (y - cy) * Z / f

X = X.flatten()

Y = Y.flatten()

Z = Z.flatten()

# Calculate the maximum value of Z and set the threshold to 98% of this value

max_Z_value = np.max(Z)

Z_threshold = 0.98 * max_Z_value

# Filter out points where Z is above the threshold

mask = Z < Z_threshold

X_filtered = X[mask]

Y_filtered = Y[mask]

Z_filtered = Z[mask]

fig = plt.figure(figsize=(15, 5))

# 3D plot

ax1 = fig.add_subplot(131, projection='3d')

ax1.scatter(X_filtered, Y_filtered, Z_filtered, c='green', marker='.')

ax1.set_xlabel('X')

ax1.set_ylabel('Y')

ax1.set_zlabel('Z')

ax1.set_xlim([-2000, 2000])

ax1.set_ylim([-2000, 2000])

ax1.set_zlim([8000, Z_threshold])

ax1.title.set_text('3D View')

# Top view (x,z)

ax2 = fig.add_subplot(132)

ax2.scatter(X_filtered, Z_filtered, c='red', marker='.')

ax2.set_xlabel('X')

ax2.set_ylabel('Z')

ax2.set_xlim([-2000, 2000])

ax2.set_ylim([8500, Z_threshold])

ax2.title.set_text('Top View (X,Z)')

# Side view (y,z)

ax3 = fig.add_subplot(133)

ax3.scatter(Y_filtered, Z_filtered, c='blue', marker='.')

ax3.set_xlabel('Y')

ax3.set_ylabel('Z')

ax3.set_xlim([-2000, 2000])

ax3.set_ylim([8500, Z_threshold])

ax3.title.set_text('Side View (Y,Z)')

plt.tight_layout()

plt.show()

def calc_focal_length(f_px, sw ,iw):

f_mm = f_px * (sw / iw)

return f_mm

focal_length = calc_focal_length(5806.559, 22.2, 3088)

print(focal_length)

# Global variables to store the trackbar values

numDisparities = 64

blockSize = 5

def on_trackbar_disp(val):

# global numDisparities

if val % 16 == 0 and val != 0:

numDisparities = val

disparity = getDisparityMap(edgesL, edgesR, numDisparities, blockSize)

# disparity = getDisparityMap(imgL, imgR, numDisparities, blockSize)

disparityImg = np.interp(disparity, (disparity.min(), disparity.max()), (0.0, 1.0))

cv2.imshow('Disparity', disparityImg)

def on_trackbar_block(val):

#global blockSize

blockSize = val

blockSize = max(5, blockSize + (blockSize % 2 == 0))

disparity = getDisparityMap(edgesL, edgesR, numDisparities, blockSize)

# disparity = getDisparityMap(imgL, imgR, numDisparities, blockSize)

disparityImg = np.interp(disparity, (disparity.min(), disparity.max()), (0.0, 1.0))

cv2.imshow('Disparity', disparityImg)

f_original = 5806.559  # Focal length in pixels for the original image size

cx_original = 1429.219  # The x-coordinate of the principal point for the original image size

cy_original = 993.403  # The y-coordinate of the principal point for the original image size

doffs_original = 114.291  # The x-difference of the principal points

baseline = 174.019  # The camera baseline in millimeters

# Original and resized image dimensions

width_original, height_original = 2960, 2016

width_resized, height_resized = 740, 505

scale_factor = width_resized / width_original

f = scale_factor * f_original

cx0 = scale_factor * cx_original

cy = scale_factor * cy_original

doffs = scale_factor * doffs_original

if __name__ == '__main__':

# Load left image

filename = 'umbrellaL.png'

imgL = cv2.imread(filename, cv2.IMREAD_GRAYSCALE)

if imgL is None:

print('\nError: failed to open {}.\n'.format(filename))

sys.exit()

edgesL = cv2.Canny(imgL, 50, 150)

# Load right image

filename = 'umbrellaR.png'

imgR = cv2.imread(filename, cv2.IMREAD_GRAYSCALE)

if imgR is None:

print('\nError: failed to open {}.\n'.format(filename))

sys.exit()

edgesR = cv2.Canny(imgR, 50, 150)

# Initialize the disparity settings

numDisparities = 64

blockSize = 5

cv2.namedWindow('Disparity', cv2.WINDOW_NORMAL)

cv2.createTrackbar('numDisparities', 'Disparity', numDisparities, 256, on_trackbar_disp)

cv2.createTrackbar('blockSize', 'Disparity', blockSize, 50, on_trackbar_block)

# Recalculate the disparity map with the new parameters

disparity = getDisparityMap(edgesL, edgesR, numDisparities, blockSize)

disparityImg = np.interp(disparity, (disparity.min(), disparity.max()), (0.0, 1.0))

cv2.imshow('Disparity', disparityImg)

plot(disparityImg, f, cx0, cy, baseline, doffs)

while True:

key = cv2.waitKey(1)

if key == ord(' ') or key == 27:

break

cv2.destroyAllWindows()

File 2:

def on_trackbar_k(val):

# global k

k = val/100

# Recalculate the disparity map with the new parameters

disparity = getDisparityMap(imgL, imgR, numDisparities, blockSize)

depth_map = cv2.divide(1.0, cv2.add(disparity, k))

disparityImg = np.interp(depth_map, (disparity.min(), disparity.max()), (0.0, 1.0))

cv2.imshow('Disparity', disparityImg)

def process_background(image, depth_map, k, blur=False, grayscale=False):

# Threshold the depth map to create a mask for the foreground

_, foreground_mask = cv2.threshold(depth_map, k, 255, cv2.THRESH_BINARY)

foreground_mask = foreground_mask.astype(np.uint8)

foreground_mask_3c = cv2.cvtColor(foreground_mask, cv2.COLOR_GRAY2BGR)

foreground = cv2.bitwise_and(image, foreground_mask_3c)

background_mask = cv2.bitwise_not(foreground_mask)

background_mask_3c = cv2.cvtColor(background_mask, cv2.COLOR_GRAY2BGR)

background = cv2.bitwise_and(image, background_mask_3c)

if blur:

foreground = cv2.GaussianBlur(foreground, (21, 21), 0)

if grayscale:

foreground = cv2.cvtColor(foreground, cv2.COLOR_BGR2GRAY)

foreground = cv2.cvtColor(foreground, cv2.COLOR_GRAY2BGR)

combined_image = cv2.add(foreground, background)

return combined_image

if __name__ == '__main__':

# Load left image

filename = 'girlL.png'

imgL = cv2.imread(filename, cv2.IMREAD_GRAYSCALE)

if imgL is None:

print('\nError: failed to open {}.\n'.format(filename))

sys.exit()

# Load right image

filename = 'girlR.png'

image = cv2.imread(filename, cv2.IMREAD_COLOR)

imgR = cv2.imread(filename, cv2.IMREAD_GRAYSCALE)

if imgR is None:

print('\nError: failed to open {}.\n'.format(filename))

sys.exit()

numDisparities = 64

blockSize = 45

k2 = 10

k = 0.99

# Create a window to display the image in

cv2.namedWindow('Disparity', cv2.WINDOW_NORMAL)

cv2.createTrackbar('numDisparities', 'Disparity', numDisparities, 256, on_trackbar_disp)

cv2.createTrackbar('blockSize', 'Disparity', blockSize, 50, on_trackbar_block)

cv2.createTrackbar('k', 'Disparity', k2, 100, on_trackbar_k)

disparity = getDisparityMap(imgL, imgR, numDisparities=numDisparities, blockSize=blockSize)

disparityImg = np.interp(disparity, (disparity.min(), disparity.max()), (0.0, 1.0))

depth_map = cv2.divide(1.0, cv2.add(disparityImg, k))

new_image = process_background(image, depth_map, k, blur=False, grayscale=True)

cv2.imshow('Image', new_image)

while True:

key = cv2.waitKey(1)

if key == ord(' ') or key == 27:

break